CAMPUS Experiment – Technical Overview for Candidates

1. Project Context

The CAMPUS (Cooperative Aggregation & Mapping for Perception in Ubiquitous Sensing) project is part of the ENVELOPE project, funded under the EU Horizon Europe programme. The goal is to validate advanced connected mobility services leveraging Beyond 5G (B5G) networking capabilities, edge computing, and network exposure APIs.

The experiment focuses on cooperative perception and dynamic HD map services, enabling connected vehicles to share and aggregate perception data in real time to improve situational awareness, particularly in complex urban scenarios.

2. High-Level System Concept

Current Advanced Driver Assistance Systems (ADAS) rely mainly on local vehicle sensors (camera, LiDAR), which are inherently limited by occlusions and line-of-sight constraints. CAMPUS extends perception beyond individual vehicles by enabling cooperative sensing.

Multiple vehicles and mobile endpoints collect perception data and send processed outputs (not raw sensor streams) to edge servers via a 5G network. At the edge, these data streams are aggregated and fused to produce a consistent and up-to-date representation of the environment.

This fused information is then integrated into a dynamic HD map, which is continuously updated with real-time semantic and safety-relevant information. The system distributes only contextually relevant map updates back to each vehicle, improving efficiency and reducing network load.

3. System Architecture

The CAMPUS architecture follows an edge-centric design and spans two main layers:

- Vehicle (Far Edge):

Each vehicle is equipped with sensors (camera, LiDAR) and an onboard computing unit (OBU), typically based on embedded GPU platforms (e.g., NVIDIA Orin). Local perception pipelines detect and track surrounding objects and generate structured outputs.

- Edge Layer:

Edge servers perform spatio-temporal fusion of perception data from multiple vehicles. They also host services for dynamic HD map generation, semantic enrichment, and relevance filtering.

The system does not rely on fixed roadside infrastructure (e.g., RSUs). Instead, it uses mobile endpoints and programmable network services.

4. Data Flow and Communication

The system operates on a continuous data loop:

- Vehicles perform local perception and generate object-level data (e.g., detected objects, positions, trajectories).

- These data are transmitted to the edge over a 5G network.

- The edge aggregates and fuses data from multiple sources.

- A dynamic HD map is updated with fused and semantically enriched information.

- Only relevant subsets of this information are sent back to each vehicle.

Communication is structured at multiple levels:

- Internal vehicle communication:

Real-time data exchange between modules (e.g., perception, tracking) may use middleware such as ROS2/DDS or equivalent mechanisms.

- Vehicle–edge communication:

Typically uses efficient binary protocols (e.g., gRPC/protobuf) for low-latency data transfer.

- Control and orchestration:

ENVELOPE APIs (REST-based) are used to manage deployment, Quality of Service (QoS), and application lifecycle.

5. Role of Network Exposure and Orchestration

A key innovation of CAMPUS is the use of programmable network capabilities via ENVELOPE APIs:

- Quality on Demand (QoD):

Enables dynamic control of network performance (e.g., latency, bandwidth) for critical data flows.

- Edge Application Management (EAM):

Supports deployment, scaling, and migration of application components across edge resources.

- Device Location API:

Provides location awareness for context-based filtering and decision-making.

These APIs allow the system to dynamically adapt to changing network conditions and mobility patterns.

6. Experimental Scenarios

The experiment is validated through multiple scenarios, including:

- Cooperative perception in occluded environments:

Vehicles share perception data to detect objects not visible locally (e.g., hidden pedestrians).

- Dynamic HD map updates:

Evaluation of latency and freshness of map updates under varying network conditions.

- Semantic enrichment:

Integration of higher-level information (e.g., intent prediction, conflict detection).

- Workload distribution:

Evaluation of different deployment strategies (e.g., processing at vehicle vs edge).

7. Key Performance Indicators (KPIs)

The system is evaluated using quantitative metrics such as:

- End-to-end latency (target ~300 ms)

- Map update freshness (~200 ms)

- Network efficiency (bandwidth usage reduction)

- Relevance filtering efficiency

Qualitative metrics include perceived safety improvements and interpretability of semantic information.

8. Expected Activities

Candidates working on this project will be involved in the following activities:

- Understanding the ENVELOPE platform and demo infrastructure:

Analyse and become familiar with the APIs and the experimental setup available at the Italian trial site (Turin), including edge infrastructure, 5G connectivity, and deployment tools.

- Studying and supporting communication mechanisms:

Investigate and evaluate suitable communication technologies for the system, including DDS and possible alternatives (e.g., gRPC, custom pipelines), with a focus on real-time performance and integration constraints.

- Development of a local testbed:

Design and implement a testbed in the lab environment that reproduces the experimental setup (vehicle–edge interaction), enabling validation and testing of communication and deployment solutions before on-site experimentation.